If you are using Ubuntu 22.04 LTS with Ettus Research USRP N210, the easiest method is to install UHD (USRP Hardware Driver) and GNU Radio from Ubuntu repositories and then verify communication with the N210. UHD is the official driver required for all USRP devices including N200/N210. (Ettus Files)

# Step 1: Update Ubuntu

sudo apt update

sudo apt upgrade -y

# Step 2: Install UHD Driver

sudo apt install libuhd-dev uhd-host -y

This installs the UHD libraries and utilities such as uhd_find_devices and uhd_usrp_probe. (Ettus Files)

# Step 3: Download UHD FPGA Images

sudo uhd_images_downloader

# Step 4: Verify UHD Installation

Check UHD version:

uhd_config_info --version

Find USRP devices:

uhd_find_devices

Probe the device:

uhd_usrp_probe

These are the recommended verification commands from Ettus. (Ettus Knowledge Base)

# Step 5: Configure Network for USRP N210

The N210 uses Gigabit Ethernet.

Connect the PC directly to the N210 and configure:

sudo ifconfig eno1 192.168.10.1 netmask 255.255.255.0

(Replace eno1 with your Ethernet interface name.)

Check connectivity:

ping 192.168.10.2

A successful ping indicates the N210 is reachable. (Ettus Knowledge Base)

# Step 6: Install GNU Radio

For GNU Radio 3.10 (Ubuntu 22.04 default):

sudo apt install gnuradio -y

Check version:

gnuradio-companion --version

# Step 7: Verify GNU Radio with UHD

Launch GNU Radio Companion:

gnuradio-companion

Add a:

- UHD USRP Source

- QT GUI Sink

Run the flowgraph.

# Step 8: Check N210 Detection

uhd_find_devices

Expected output:

-- UHD Device 0

Device Address:

type: usrp2

addr: 192.168.10.2

name: N210

# Step 9: Useful Commands

Show device information:

uhd_usrp_probe

Benchmark receive:

benchmark_rate --rx_rate 1e6

Benchmark transmit:

benchmark_rate --tx_rate 1e6

# If UHD Cannot Detect N210

Check:

- Ethernet cable connected.

- PC IP = 192.168.10.1.

- N210 IP = 192.168.10.2.

- Firewall disabled.

- Ping works before running UHD commands.

# One-Line Installation

sudo apt update && sudo apt install -y gnuradio libuhd-dev uhd-host && sudo uhd_images_downloader

Please tell me:

- Ubuntu version (20.04, 22.04, or 24.04)

- GNU Radio version required (3.8, 3.10, or 3.11)

- Whether your USRP N210 has a WBX, SBX, or XCVR daughterboard

and I can provide a complete lab setup guide with screenshots and troubleshooting steps.